##### Connecting the Paging Data Terminal

The PDT has four standard connectors on its back face plate plus, if fitted, an additional three optional connectors.

###### Standard Connectors

25 pin DB25 (P1) - Standard centronics parallel printer output.

9 pin DB9 (S1) -  Standard RS232 serial port for programming and data I/O (pinout below.)

BNC connector is for connecting the external antenna.

DC Socket - Power in (9-32v AC or DC (recommended 12v DC).

###### Optional Connectors

An RJ11 4 pin socket (S2) provides a second serial port for multi I/O requirements.

2 RJ45 8 pin sockets (R1 & R2) provide connection for up to 4 relays. Either port may also be used as an alternative means to connect power input. (See pinout below)

General Specifications

Receiver type	Synthesised FSK FM receiver

Freq. bands available	135-174, 408-412,450-470, 929-932 MHz (5MHz splits), 25kHz band width

Sensitivity	Typically better than -118 dBm into 50 ohms

Decoding format 	Pocsag or FlexTM

No. Capcodes/Addresses	Large (Varies according to format selected and other functions)

LCD Display Format	Sunlight readable, Blue/White Graphical 240x64,  LED backlight

Operating Temp 	0-70degC

Display area	127x 34mm (5”x1.5”) 8 or 4 lines text

Antenna	External on BNC (SMA for 900MHz)

Physical	227x75x42mm (~9x3x1.5”) excluding bracket and antenna connector

Power	10-32 VDC or AC

Consumption	Up to 500mA on start up, 100-250 continuous

Relays	Switch contact ratings 0.5A @125VAC,  1A @24VDC resistive load

Approvals	FCC CFR 47 Part 15, C-Tick

# PDT2000 - PAGING DATA TERMINAL USER INSTRUCTIONS

##### Receiving & Reading Messages

On receipt of a message the PDT will beep and an unopened envelope  will appear at the top of the LCD display.  Press the green Acknowledge key (A) to display the message and the down key to scroll if the message is longer than one screen.  You may use the left and right keys to scroll between different messages.

Once the message has been read the symbol will change to an opened envelope .

The PDT2 will hold up to 40 messages in its standard message stack and up to 10 locked messages.  Locked messages appear at the bottom left of the display thus .

To return to the main screen press the red Mode (M)  key once.

##### Control Functions

To activate the control functions  press the red Mode (M)  key once.

The following symbols will appear:

As the cursor is moved across the symbols a text description will appear below them.

Exit 	Return to main screen

Delete Message 	Delete the 1 message under which the message cursor is placed.

Delete All Read Messages	Delete all messages that have been read ().

Delete All Messages	Delete all messages except locked messages.

Lock Message	Place the selected messages in the Locked Messages section.  Note the same symbol is used to unlock locked messages.

Display Functions	Adjust LCD Contrast and Backlight Brightness.  Use Up/Down to select and Left/Right keys to adjust

Print Message	Print the selected message.  Note the same may be achieved by pressing the blue Print (P) key

System Setup	Set various display,  message, alerting and date/time functions.  See below:

##### System Setup*

Press the red Mode (M) key and move the cursor to the System Setup  selection.  Press M again and use Up/Down keys to select menu are.  Press the Right key to select the Options Menu.  The Left and Right keys are used to change the selection.  The A key will return you to the previous screen and the M key will return you to the main screen.

| ###### Display Options |  |  |
| --- | --- | --- |
| ###### Function | Description | Settings |
| ###### Backlight Time | Set how long backlight is to remain on after a key press or message received (see next option) | 5s/10s/30s/1m/5m/ Permanent |
| ###### Backlight on Msg | Turns the backlight on when a message is received. | Y or N |
| ###### Display Instantly | Display message automatically when it is received | Y or N |
| ###### Display For | Sets how long the message will remain on display | 5s/10s/30s/1m/5m/Perm |
| ###### Display Font Size | Changes Text size & no. of lines of text on display - | Small (8), Med (6), Lge (4) |
| ###### Scroll | Scroll text in message 1 or 3 lines at a time | 1 or 3 |
| ###### Show [Name] | Display the name assigned to the cap code/address | Y or N |
| ###### Show Capcode | Display Capcode at the beginning of the message. | Y or N |
| ###### Show [Number] | Display The number of the message 1-40 | Y or N |
| ###### Date Format | Display dates mm/dd/yy or dd/mm/yy | US or International |
| ###### Time Format | 12 or 24 hour | 12 or 24 |
| ###### Delete All Read Msgs | Turns the Delete All Read Messages option on and off. | Y or N |
| ###### Delete All | Turns the Delete All Messages option on and off | Y or N |
| ###### Lockup Msg Option | Allows messages to be locked so they will not be deleted | Y or N |
| ###### Set Time Option | Allows user to set time through Settings menu | Y or N |
| ###### P Key to Delete All |  | Y or N |
| ###### Display Maildrop | Instantly displays maildrop messages | Y or N |
| ###### Max No. Messages | Sets the max no. off messages to display on PDT | 1-5 & 10-40 in steps of 5 |

| ###### Message/Alert Options |  |  |
| --- | --- | --- |
| Function | Description | Settings |
| Audible Alert | Beeper sounds on receipt of message | On or Off |
| Beep Continuously | Beeper sounds continuously until any key is pressed | Y or N |
| Reminder Beep | Reminder beep sounds until message is read | On or Off |
| Initial Alert Time | Time beeper sounds on initial receipt of message | 1s/5s/10s/30s/1m/ 5m |
| Warning Initial Time | Time beeper sounds on receipt of Warning type message* |  |
| Watch Initial Time | Time beeper sounds on receipt of Watch type message* |  |
| Reminder Interval | How often the reminder beep sounds | 30s/1m/2m/3m/4m/5m |
| Reminder Period | How long reminder beep continues for | 5m/10m/15m/30m/Forever |
| Combine Duplicate Msg | Identical messages displayed as separate messages | Y or N |
| Relay Lockout | Turns Relay lockout facility on or off | On or Off |
| Relay Lockout Period | Period during which Relays will not respond to identical commands on the same capcode or message. | 1s-5s/10-30s/1m-5m 10m-30m |
| Timestamp message | Place the date and time of receipt at end of message | Y or N |
| *Warning/Watch | Special alert types set when capcode is programmed |  |

| ###### Set Time & Date | Displays date and time in selected format | Use Up/Down keys to adjust L/R to change selection |
| --- | --- | --- |

*Note:  From Ver2.4 on these settings may be set in PDTP and locked out of the user interface.

This product is manufactured in New Zealand by:

WiPath Communications Ltd,  PO Box 8798, Symonds St, Auckland

3/2 Haultain St, Eden Tce, Auckland New Zealand,  Ph: +64-9-3021142,

Fax: +64-9-3021148	Email:  info@wipath.co.nz	www.wipath.co.nz

US: Wipath Communications LLC, 4467 Terracemeadow Ct. Moorpark, CA93021

Ph/Fax: +1-805-532-9964,	Email:  info@wipath.com	www.wipath.com

Australia: WiPath Communications P/L, PO Box 6947, Silverwater, NSW 2128

44/8 Avenue of Americas, NSW 2127.   Ph: 61-2-8004 0535

Fax 61-2-9647 15591	Email:  info@wipath.com.au	www.wipath.com.au

Distributed By: